Chemical Compatibility Chart

| RGN | Reactivity Group | Incompatible With: |
| --- | --- | --- |
| 1 | Acids, Mineral, Non-Oxidizing | 4-15,17-26,28,30-34,101-107 |
| 2 | Acids, Mineral, Oxidizing | 3-34,101-103,105-107 |
| 3 | Acids, Organic | 2,4,5,7,8,10-12,15,18,21,22,24,265,33,34,102-105,107 |
| 4 | Alcohols and Glycols | 1-3,8,18,21,25,30,34,104,105,107 |
| 5 | Aldehydes | 1-3,7,8,10,12,21,25,27,28,30,33,34,104,105,107 |
| 6 | Amides | 1,2,21,24,104,105,107 |
| 7 | Amines, Aliphatic and Aromatic | 1-3,5,12,17,18,21,24,30,34,104,105,107 |
| 8 | Azo Compounds, Diazo Compounds and Hydrazines | 1-5,9,11-13,17-23,25,30-34,102-107 |
| 9 | Carbamates | 1,2,8,10,21,22,25,30,104,107 |
| 10 | Caustics | 1-3,5,9,13,17-19,21,22,24-27,32,34,102,103,107 |
| 11 | Cyanides | 1-3,5,9,13,17-19,21,22,24-27,32,34,102,103,107 |
| 12 | Dithiocarbamates | 1-3,8,17-19,21,25,30,34,103,104,107 |
| 13 | Esters | 1,2,8,10,21,25,102,104,105,107 |
| 14 | Ethers | 1,2,104,107 |
| 15 | Fluorides, Inorganic | 1-3,107 |
| 16 | Hydrocarbons, Aromatic | 2,104,107 |
| 17 | Halogenated Organics | 1,2,7,8,10,11,20-23,25,30,104,105,107 |
| 18 | Isocyanates | 1-4,7,8,10-12,20-22,25,30,31,33,104-107 |
| 19 | Ketones | 1,2,8,10,11,20,21,25,30,104,105,107 |
| 20 | Mercaptons and Other Organic Sulfides | 1,2,8,17-19,21,22,25,30,34,104,105,107 |
| 21 | Metals, Alkali and Alkaline Earth Elemental | 1-13,17-20,25-27,30-32,34,101-104,106,107 |
| 22 | Metals, Other Elemental and Alloys as Powders, Vapors or Sponges | 1-3,8-10,17,18,20,28,30,34,102-104,106,107 |
| 23 | Metals, Other Elemental and Alloys as Sheets, Rods, Drops, Moldings | 1,2,8,17,102-104,107 |
| 24 | Metal and Metal Compounds, Toxic | 1-3,6,7,10,26,30,34,102,103,106,107 |
| 25 | Nitrides | 1-5,8-13,17-21,26-27,30,31,34,101-104,106,107 |
| 26 | Nitrites | 1-3,10,21,24,25,30,104,105,107 |
| 27 | Nitro Compounds, Organic | 2,5,10,21,25,104,105,107 |
| 28 | Hydrocarcons, Aliphatic, Unsaturated | 1,2,5,22,30,104,107 |
| 29 | Hydrocarbons, Aliphatic, Saturated | 2,104,107 |
| 30 | Peroxides and Hydroperoxides, Organic | 1,2,4,5,7-9,11,12,17-22,24-26,28,31-34,101-105,107 |
| 31 | Phenols and Cresols | 1,2,8,18,21,25,30,34,102-105,107 |
| 32 | Organophosphates, Phosphothioates, Phosphodithioates | 1,2,8,10,21,30,34,104,105,107 |
| 33 | Sulfides, Inorganic | 1-3,5,8,18,30,34,102-104,106,107 |
| 34 | Epoxides | 1-5,7,8,10-12,20-22,24,25,30-33,102,104,105,107 |
| 101 | Combustible and Flammable Materials, Misc. | 1,2,21,25,30,102,104,105,107 |
| 102 | Explosives | 1-3,8,10,13,21-25,30,31,33,34,101,105-105,107 |
| 103 | Polymerizable Compounds | 1-3,8,10-12,21-25,30,31,33,102,104,105,107 |
| 104 | Oxidizing Agents, Strong | 1,3-9,11-14,16-23,25-34,101-103,105,107 |
| 105 | Reducing Agents, Strong | 1-8,12,13,17-20,26,27,30,31,32,34,101-104,106,107 |
| 106 | Water and Mixtures Containing Water | 1,2,8,18,21,22,24,25,33,105,107 |
| 107 | Water Reactive Substances | ALL! |

I. Acids, Mineral, Non-Oxidizing

4. Alcohols and Glycols 
5. Aldehydes 
6. Amides 
7. Amines, Aliphatic and Aromatic 
8. Azo Compounds, Diazo Compounds, and Hydrates 
9. Carbamates 
10. Caustics 
11. Cyanides 
12. Dithiocarbamates 
13. Esters 
14. Ethers 
15. Fluorides, Inorganic 
17. Halogentated 
18. Isocyanates 
19. Ketones 
20. Mercaptons and Other Organic Sulfides 
21. Metals, Alkali and Alkaline Earth Metal 
22. Metals, Other Elemental and Alloys as Powders, Vapors or Sponges 
23. Metals, Other Elemental and Alloys as Sheets, Rods, Drops, Moldings 
24. Metal and Metal Compounds, Toxic 
25. Nitrides 
26. Nitrites 
28. Hydrocarbons, Aliphatic, Unsaturated 
30. Peroxides and Hydrocarbons 
31. Phenols and Cresols 
32. Organophosphates, Phosphothioates, Phosphodithioates 
33. Sulfides, Inorganic 
34. Epoxides 
101. Combustible and Flammable Materials, Misc. 
102. Explosives 
103. Polymerizable Compounds 
104. Oxidizing Agents, Strong 
105. Reducing Agents, Strong 
106. Water and Mixtures Containing Water 
107. Water Reactive Substances

II. Acids, Mineral, Oxidizing

3. Acids, Organics 
4. Alcohols and Glycols 
5. Aldehydes 
6. Amides 
7. Amines, Aliphatic and Aromatic 
8. Azo Compounds, Diazo Compounds, and Hydrates 
9. Carbamates 
10. Caustics 
11. Cyanides 
12. Dithiocarbamates 
13. Esters 
14. Ethers 
15. Fluorides, Inorganic 
16. Hydrocarbons, Aromatic 
17. Halogentated 
18. Isocyanates 
19. Ketones 
20. Mercaptons and Other Organic Sulfides 
21. Metals, Alkali and Alkaline Earth Metal 
22. Metals, Other Elemental and Alloys as Powders, Vapors or Sponges 
23. Metals, Other Elemental and Alloys as Sheets, Rods, Drops, Moldings 
24. Metal and Metal Compounds, Toxic 
25. Nitrides 
26. Nitrites 
27. Nitro Compounds, Organic 
28. Hydrocarbons, Aliphatic, Unsaturated 
29. Hydrocarbons, Aliphatic, Saturated 
30. Peroxides and Hydrocarbons 
31. Phenols and Cresols 
32. Organophosphates, Phosphothioates, Phosphodithioates 
33. Sulfides, Inorganic 
34. Epoxides 
101. Combustible and Flammable Materials, Misc. 
102. Explosives 
103. Polymerizable Compounds 
105. Reducing Agents, Strong 
106. Water and Mixtures Containing Water 
107. Water Reactive Substances

- Alcohols and Glycols

1. Acids, Mineral, Nonoxidizing 
2. Acids, Mineral, Oxidizing 
3. Acids, Organic 
8. Azo Compounds, Diazo Compounds and Hydrazines 
18. Isocyanates 
21. Metals, Alkali and Alkaline Earth Metals 
25. Nitrides 
30. Peroxides and Hydroperoxides, Organic 
34. Epoxides 
104. Oxidizing Agents, Strong 
105. Reducing Agents, Strong 
107. Water Reactive Substances

- Aldehydes

1. Acids, Mineral, Nonoxidizing 
2. Acids, Mineral, Oxidizing 
3. Acids, Organics 
7. Amines, Aliphatic and Aromatic 
8. Azo Compounds, Diazo Compounds, and Hydrates 
10. Caustics 
12. Dithiocarbamates 
21. Metals, Alkali and Alkaline Earth Metal 
25. Nitrides 
27. Nitro Compounds, Organic 
28. Hydrocarbons, Aliphatic, Unsaturated 
30. Peroxides and Hydrocarbons 
33. Sulfides, Inorganic 
34. Epoxides 
104. Oxidizing Agents, Strong 
105. Reducing Agents, Strong 
107. Water Reactive Substances

- Amides

1. Acids, Mineral, Nonoxidizing 
2. Acids, Mineral, Oxidizing 
21. Metals, Alkali and Alkaline Earth Metal 
24. Metal and Metal Compounds, Toxic 
104. Oxidizing Agents, Strong 
105. Reducing Agents, Strong 
107. Water Reactive Substances

- Amines, Aliphatic and Aromatic

1. Acids, Mineral, Nonoxidizing 
2. Acids, Mineral, Oxidizing 
3. Acids, Organics 
5. Aldehydes 
12. Dithiocarbamates 
17. Halogentated 
18. Isocyanates 
21. Metals, Alkali and Alkaline Earth Metal 
24. Metal and Metal Compounds, Toxic 
30. Peroxides and Hydrocarbons 
34. Epoxides 
104. Oxidizing Agents, Strong 
105. Reducing Agents, Strong 
107. Water Reactive Substances